|  | BEFORE HAUL OUT: - Top off Diesel Tank before hauling- add fuel stabilizer & Biocide - Pump seawater through heads overboard- clean out all lines - Remove Sails - Remove all Soda- diet/w/o sugar will freeze & explode - Remove all spare propane bottles (Grill) - PULL SPEED SENSOR- or it will be crushed by the travel lift! - RV Antifreeze- Used 12 gallons winter 2021 |
| --- | --- |
|  | ENGINE (2 gal) - Change Engine Oil & Filter (while hot) Cap 6-7 Quarts Hoses - Change Transmission Fluid Cap ~1.5 Quarts Dextron III Mercon - Don’t screw in dipstick to read level, use paper towel to read level. - Change Fuel Filters Engine (every 2 years: 2015/2017) - Drain water from Racor Filter - Air intake- clean - Pull Strainer on raw water Strainer & Clean, Vacuum out water - Check: Belts, Hoses, Motor Mounts - Pull Raw Water impeller **REPLACED SPRING 2021** - Pull Raw Water Intake- Flush with 2 Gal of Glycol Antifreeze - Check exhaust - Leave disconnected until finished with A/C system - Blow out seacock with air while opening/closing - Drain Seacock (remove plug) - Plug Exhaust port (transom) with Rag - batteries- Charge and Then Disconnect |
|  | Water Tanks (4 Gal ALL without engine) - Drain Fwd, Port and Stbd tanks using 12v pump. - Vacuum out Port and Stbd Tanks to remove remaining water. - Remove fresh water strainer & cover- Apply shop vac here and open up P&S & FWD tank lines to remove all water in the lines between the tanks and the manifold. - Use Air Compressor to blow out water in lines/plug in adapter/bypass pump - Use air to empty water heater- open drain on aft end of Water Heater use pex/hose to galley sink. Blow out remaining water while red bypass valve is open. - Put Hot Water Heater line into bypass mode. - Clean strainers for Fresh water and sump - Close Port & Stbd valves, open Center valve for Forward tank. - Using the deck fill, fill forward water tank with 4 Gallons of antifreeze. - Unplug Water Heater, Plug in Engine Room Heater - Switch water pump on and Run water to - Aft Head and Shower until Pink, Fwd Head until Pink - Galley sink until Pink - Let some into the water heater (via bypass valves) |
|  | A/C Unit (3/4 gal) - Option 1 - NEW 2015 (could not get to work fall 2017) - Pull cap off “T” under Nav Station - Use ¾” hose to blow water out of pump/strainer/seacock - Close seacock and blow water out of A/C Unit - May not have to use antifreeze in A/C Unit. - Option 2 (used fall 2021) - Pull A/C Coolant Hose - Put Funnel into hose and Run Pump to circulate Antifreeze (3/4 gal) - Leave running & pull and clean the strainer - Blow out seacock with air while opening/closing valve - Reconnect the hose and then turn off the AC unit. (Do this QUICKLY). |
|  | Fwd Seacock intake to Washdown Pump (1/2 gal) - Pull hose & Run Washdown Pump until pink. (1/2 gal) - Clean strainer in fwd anchor locker. - Blow out seacock with air while opening/closing valve- leave closed. 5/8” - Drain Seacock (remove plug) |
|  | Fwd Head- (2 gal) - Turn Diverter valve to Overboard (~1 ½ gal) - Pump fast until air comes out of seacock (open and close seacock quickly to get all water out of seacock). - Note: If this works then you only need 1 GAL. If not two. - Put ½ gal Antifreeze into bowl and pump overboard w/air - Drain Seacock (remove plug) make sure you see red antifreeze - Turn Diverter valve to TANK (~ ½ Gal) - Put funnel into water intake on head (remove plug) - Pump (½- 1 gal?) Antifreeze thru head into holding tank. |
|  | Aft Head- (1-2 Gal) - Turn Diverter valve to TANK (normal position) - Put funnel into raw water intake for head. - Pump (1 gal) Antifreeze thru head into holding tank. - Blow out raw water intake for head with hose/mouth. - Remove seacock drains for intake and overboard. - ONLY IF DIVERTER WAS SET TO OVERBOARD THAT SEASON: - Turn Diverter valve to Overboard - Pump fast until air comes out of seacock (open and close seacock quickly to get all water out of seacock). - Put 1 gal Antifreeze into bowl and pump overboard w/air - Drain Seacock (remove plug) make sure you see red antifreeze - Drain raw water intake Seacock (remove plug) Refrigerator Drain Antifreeze into Refrigerator Drain (Use Antifreeze from Galley Sink) |
|  | Galley Sink - Metal tape on port side cockpit drain to plug drain. - Disconnect Engine room bilge pump hose & plug opening - Rag & Shop vac on sink drain, blow air into seacock under sink and open and close port cockpit drain a few times to remove water. - Leave port cockpit seacock closed - Cycle seacock under sink to ensure it is drained - Remove fuse for Aft bilge pump & reconnect drain line. Misc - Cockpit Drain Seacocks in Engine room- Exercise and leave starboard seacock open. Port seacock stays closed (to sink/engine bilgepump) - Remove Bimini & Tie up supports - Pull engine hoist and install plug on top - Remove davits, leave in cockpit - Wash down Deck & boat- check for any leaks – ports/etc - Winter Cover- Install & Tie Lines OUTSIDE cover to keep it from inflating - Re-Energize moisture absorbent pouches |
|  | If Pulling the Mast - Remove Windex & Wind Instruments at masthead, - Remove VHF Antenna - Disconnect all electrical connections at mast step. |

|  | BEFORE HAUL OUT LOAD ON BOAT: - Shop Vac, Hoses, Connector, Extra Filter - 300’ garden hose & nozzle to wash down boat - 100’ extension cord - ladder & lock (chain) - Anti-freeze bottles - oil changer pump - oil container for old oil - oil for oil change & filters - winter cover and supports (PVC Tubing and wood base) - rug to cushion boat in slings - Blower to clean off Decks - Tool Pouch w/hammer, large punch to remove centerboard - 4’ plastic Level (Attach to rudder to level boat) |
| --- | --- |

|  | Parts Needed for Winterization: |
| --- | --- |
|  | Fuel Filters (Yanmar P/N 129470-55701 & Racor p/n 2010SM) |
|  | Oil Filter (Yanmar p/n 129150-35150) |
|  | 2 Qts ATF Dextron III Mercon |
|  | 2.0 Gal 15w-40W Oil |
|  | 12 Gal Glycol Antifreeze (Non-Toxic) used 12 gal 2014 |
|  | Fuel Stabilizer |

|  | Spring Cleanup: |
| --- | --- |
|  | Grease Max Prop & Install new Zinc on prop and two 1 ¼” Zinc’s on the shaft |
|  | Spray Antifoulant on Prop |
|  | Paint Bottom Ablative (1 gal just covers) 2007- three Gals gave two coats |
|  | Three gallons of Micron Extra gave two coats & to centerboard w/ ½ gal left |
|  | Used ¼” Nap roller for boat. 9x6 Green Scrub Pads to wet sand hull |
|  | Wax Boat |
|  | Bring to Boat before launching: |
|  | 6 Rug strips for travellift (Oak Harbor has good pads) |
|  | Davits (attic garage) |
|  | Dink |
|  | Sails |
|  | GPS, Binoculars – any electronics taken off the boat for the winter |
|  | Prep for Launch: |
|  | Reconnect Batteries |
|  | Remove Plug Exhaust port (transom) Rag |
|  | Check Transmission Fluid Cap ~1.5 Quarts Dextron III Mercon |
|  | Check Engine Coolant |
|  | Check Engine Oil |
|  | Check Raw Water Impeller (replace/reinstall) replace spring 2015 |
|  | Clean Raw Water Strainer washdown pump |
|  | Check Belt Tension Alternator/Water Pump |
|  | Check Stuffing Box/ PSS Shaft Seal for wear |
|  | Spare Propane back to boat (Grill) |
|  | Cushions Back to boat |
|  | Sails on board |
|  | GPS / Binoculars / VHF / Charts |
|  | Dive Gear back on board |
|  | Just before launch: |
|  | Pull Backstay for TravelLift (Not Necessary at Oak Harbor) |
|  | Dink on davits |
|  | Crank up Centerboard |
|  | Close all seacocks |
|  | . |
|  | After Launch: |
|  | Burp PSS Shaft Seal (get air out) |
|  | Flush Water tanks |
|  | Replace water filter |
|  | Hose out Bilge and Flush |
|  | Light lubrication on Turnbuckles |
|  | Check Rigging Tension |